=====================================================

=                                                   =

=      Alcormicro Smart card reader                 =

=    Linux TARBALL Package Release Note             =

=					            =

=====================================================

=========================================

Step1. Install  pcsc-lite

$ PKG_CONFIG_PATH=$PKG_CONFIG_PATH:/usr/local/lib/pkgconfig;export PKG_CONFIG_PATH

$ tar xjf pcsc-lite-1.7.4.tar.bz2

# cd pcsc-lite-1.7.4

# ./configure

# make

# make install          (Login as a root to install driver)

Step2. Install ccid driver

# tar xjf ccid-1.4.5-alcor.tar.bz2

# cd ccid-1.4.5-alcor

# ./configure --enable-twinserial --disable-libusb

# make

# make install          (Login as a root to install driver)

configuring the driver for the serial reader

~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~~

You have to create or edit the file /etc/reader.conf. The file should

contain something like:

# AlocrMicro reader with serial communication

#  - reader is the reader name. It is needed for multi-slot readers.

#    Possible reader values are: AU9540, AU9528,

#    AU9540 (default value)

# example: /dev/ttyS0:AU9540

#FRIENDLYNAME      "Generic serial"

#DEVICENAME        /dev/ttySn[:reader]

#LIBPATH           TARGET

#CHANNELID         n

# gedit /usr/local/etc/reader.conf.d/libccidtwin

#AU9540

FRIENDLYNAME      "Generic serial"

DEVICENAME        /dev/ttyS1:AU9540

LIBPATH           /usr/local/lib/pcsc/drivers/serial/libccidtwin.so

CHANNELID         1

or

#AU9528

FRIENDLYNAME      "Generic serial"

DEVICENAME        /dev/ttyS1:AU9528

LIBPATH           /usr/local/lib/pcsc/drivers/serial/libccidtwin.so

CHANNELID         2

note: you can test it follow these steps

# cd pcsc-lite-1.7.2/doc/example

# ./pcsc_demo