RTL8188EUS EEPROM Content

Date: 2012/06/26

Version: R03

This document is subject to change without notice. The document contains Realtek confidential information and must not be disclosed to any third party without appropriate NDA.

# EEPROM (eFuse) Contents

The RTL8188EUS is embedded an internal non-volatile memory called eFuse. Values in the eFuse allow default fields in PCI configuration space and I/O space to be overridden following an internal power on reset, or software eFuse auto-load command. The RTL8188EUS will auto-load values from the eFuse to these fields in configuration space and I/O space.

The eFuse emulates the structure of a usual EEPROM such as 93C46. We will describe the content and its addressing of the eFuse as we did in 93C46 and will mix the terms of EEPROM and eFuse in the following text. After the initial power on or auto-load command to the eFuse, the RTL8188EUS performs a series of EEPROM read operations from the EEPROM addresses 00h to 7Fh. The definition of each EEPROM byte is shown as the below.

Note: It is suggested to obtain Realtek approval before any change on the default settings of the EEPROM.

Table 1 RTL8188EUS EEPROM and eFuse CONTENTS

| Bytes | Contents | Description | Default |
| --- | --- | --- | --- |
| 00h | 29h | These 2 bytes contain the ID code word for the RTL8188EUS. The RTL8188EUS will load the contents of the EEPROM into the corresponding location if the ID word is correct. | 29h |
| 01h | 81h |  | 81h |
| 02h~0Fh | Reserved | Reserved for Realtek. Do not change this field without Realtek’s approval | -- |

| Bytes | Contents | Description | Default |
| --- | --- | --- | --- |
| 10h | Path A 2.4G CCK-1TX Power Index (Absolute Value) | Path A CCK Power Index for Ch 1,2, Range 0~63. | 2Dh |
| 11h |  | Path A CCK Power Index for Ch 3, 4, 5, Range 0~63. | 2Dh |
| 12h |  | Path A CCK Power Index for Ch 6, 7, 8, Range 0~63. | 2Dh |
| 13h |  | Path A CCK Power Index for Ch 9, 10, 11, Range 0~63. | 2Dh |
| 14h |  | Path A CCK Power Index for Ch 12, 13, Range 0~63. | 2Dh |
| 15h |  | Path A CCK Power Index for Ch 14, Range 0~63. | 2Dh |
| 16h | Path A 2.4G BW40-1S TX Power Index (Absolute Value) | Path A 2G BW40-1S Power Index for Ch 1, 2, Range 0~63. | 2Dh |
| 17h |  | Path A 2G BW40-1S Power Index for Ch 3, 4, 5, Range 0~63. | 2Dh |
| 18h |  | Path A 2G BW40-1S Power Index for Ch 6, 7, 8, Range 0~63. | 2Dh |
| 19h |  | Path A 2G BW40-1S Power Index for Ch 9, 10, 11, Range 0~63. | 2Dh |
| 1Ah |  | Path A 2G BW40-1S Power Index for Ch 12, 13, 14 Range 0~63. | 2Dh |
| 1Bh | Path A 2.4G BW20-1S TX Power Index Difference OFDM-1 TX Power Index Difference | Power Index Difference between BW20-1S and BW40-1S. Bit[7:4] : Path A 2G Offset, Range –8~7. Power Index Difference between OFDM-1Tx and BW40-1S. Bit[3:0] : Path A 2G Offset, Range –8~7. | 02h |
| 1Ch ~ B7h | Reserve | - | - |
| B8h | Channel Plan | Bit[7]: Software configure mode 0h: Enable software configure( refer to Channel Plane Domain Code) 1h: Disable software configure( can't change Channel Plan Setting) Bit[6:0]: Channel Plan Domain Code; eFuse Value; Channels; Description. 2G_WORLD; 20h; 1~13; Worldwird 13. 2G_ETSI1; 21h; 1~13; Europe 2G. 2G_FCC1; 22h; 1~11; US 2G. 2G_MKK1; 23h; 1~13, 14; Japan 2G. 2G_ETSI2; 24h; 10~13; France 2G. <2G>_Global; 41h; 1~13, 14; Global domain. | 20h |
| B9h | Crystal Calibration | XTAL_K Value Bit[5:0], Xi=Xo Range 0~3F h. Bit[7:6]: reserved FF h = 00 h | 20h |
| BAh | Thermal Meter | Thermal Meter Default Value System maker will calibrate a value and save it in EEPROM. Bit[7:0]: Thermal Meter Value | 1Ah |
| BBh | Reserve | Reserved for Realtek. Do not change this field without Realtek’s approval. | 00h |
| BCh | 2G PA Type | 2G PA Bit[7]: Reserved Bit[4]: Path-A Internal/External PA 0h: Internal PA 1h: External PA Bit[3:0]: Reserved | 00h |
| BDh | 2G LNA Type and Gain Selection | Bit[2:0]: 2G path-A external LNA Gain, used to modify DIG mechanism 0h~7h: External LNA, 8~22dB with 2dB/step Bit[3]: 2G Path-A Internal/External LNA 0h: Internal LNA 1h: External LNA Bit[7:4]: Reserved | 00h |
| BEh ~ C0h | Reserve | - | - |
| C1h | Board Options | Bit[2:0]: reserved Bit[3]: Non-interrupt Antenna Diversity 0: disable 1: enable Bit[4]: Reserved Bit[7:5]: Board Type (PCIe) 0h: WiFi solo-mCard 1h: WiFi+BT combo-mCard 2h: PCIe Card 3h~7h: Reserved. | 00h |
| C2h | Feature Options | Bit[1:0]: function configuration of pin_LED0 and pin_LED1 Bit[3:2]: Link Speed shown in OS 0h: Current Tx PHY Rate 1h: Current Rx PHY Rate 2h: Maximum RX PHY Rate 3h: reserved Bit[4]: power down mode selection 0: radio off 1: power down Bit[5]: Enable bluetooth coexistence 0: Disable 1: Enable Bit[6]: Enable WoWLAN 0: Disable 1: Enable Bit[7]: Enable WAPI support 0: Disable 1: Enable | 00h |
| C3h | BT Setting | Bit[0]: Total antenna number 0: 2-Antenna (default) 1: 1-Antenna Bit[7:1]: Reserved | 10h |
| C4h | Version | The EEPROM content version. | 00h |
| C5h | Customer ID | Customer ID (0x00 and 0xFF are reserved for Realtek) | 00h |
| C6h | Reserved | Reserved for Realtek. Do not change this field without Realtek’s approval. | 00h |
| C7h | Reserved | - | - |
| C8h | Reserved | Reserved for Realtek. Do not change this field without Realtek’s approval. | 00h |
| C9h | T/Rx Antenna Options | Bit[7:0] 00 h : reserved 01 h : for RTL8188EUS, 1Tx and 1RxCG are diversity.(2 Ant with SPDT) 02 h : for RTL8188EUS, 1Tx and 2Rx are diversity.( 2 Ant, Tx and RxCG are both on aux port, RxCS is on main port ) 03 h : for RTL8188EUS, 1Tx and 1RxCG are fixed.(1Ant, Tx and RxCG are both on aux port) 04 h ~ FE h : Reserved FF h : (default) not use. | 01h |
| CAh ~ CFh | Reserved | - | - |
| D0h ~ D1h | VID | USB Vendor ID | 0BDAh |
| D2h ~ D3h | PID | USB Product ID | 8179h |
| D4h | USB optional function_0 (SIE) | Bit[0]: Serial number Bit[1]: USB remote wakeup function Bit[2]: Device power(Self power) Bit[3]: Earliest time when downstream port assert Chirp K. Bit[4]: Chirp K duration Bit[5]: RSVD Bit[6]: FS/HS Time Out check when set Bit[7]: Disable FS qual DESC | 42h |
| D5h | USB optional function_1 (SIE) | Bit[0]: Link Power Management(LPM) support (0 : Disable, 1 : Enable) Bit[3:1] : Suspend Timing (default 011b:suspend timing = 3.05ms) Bit[4] : RSVD Bit[5] : SuperSpeed USB Device Capability Descriptor support selection (0:Disable, 1:Enable) Bit[6] : Response to ACK or NYET if HIRD value should be less than 300us. (0:NYET, 1:ACK) Bit[7] : Always response to NYET packet | 66h |
| D6h | TXQ, RXQ, INTQ EP setting (SIE) | Bit[1:0] : 00 : 3EP - RxQ + TXQ0 + TXQ1, 01 : 4EP - RxQ + INTQ + TXQ1 + TXQ2, 10 : 5EP - RxQ + INTQ + TXQ1 + TXQ2 + TXQ3. Bit[7:2]: RSVD | 00h |
| D7h ~ DCh | MAC address | WiFi MAC address | - |
| DDh~116h | Manufacture String & Product String | Manufacture String & Product String | - |
| 117h~12Fh | Reserved |  | - |
| 130h~13C | Reserved | Reserved for Realtek. Do not change this field without Realtek’s approval | -- |

Revision history

| Revision | Editor | Release Date | Description |
| --- | --- | --- | --- |
| R01 | Seam | 2012/06/07 | First release |
| R02 | Mike | 2012/06/12 | Add “2G_Global” option for channel plan defined in B8h |
| R02 | Edlu | 2012/06/26 | update 0x1B = 0x02 |